- G33 Appendix 1- ASBESTOS ALERT FOR MAINTENANCE WORKERS

- ASBESTOS DUST KILLS

Thousands of people die each year in the UK from asbestos-related diseases, because they inhaled asbestos dust many years ago.  Victims include people who worked in the building trades, such as Joiners, Plumbers, and Electricians etc.  There is no cure for these diseases and if you undertake maintenance, repair or refurbishment works on older buildings, you could be at risk.

- THE DANGER

Thousands of tons of asbestos are still in place in older buildings.  The danger to you comes from drilling, cutting, sanding or disturbing materials made from asbestos and breathing in the dust.

- YOU WILL FIND ASBESTOS

- as a sprayed insulating coating on steelwork and concrete etc

- as lagging on pipes and boilers etc

- as insulation board in walls, on doors and ceilings

- as asbestos cement for roof and wall coverings, pipes and tanks etc

- as asbestos textiles, gaskets, ropes and woven textiles

- in some decorative plasters, e.g. old 'Artex'

- WHAT YOU SHOULD DO

- Ask whether there has been a check for asbestos where you are working?

- If there hasn’t been a check, treat any insulation or insulating board etc as asbestos.

- If you are uncertain how to proceed, stop work and seek advice.

- Use the safe work methods and equipment your employer provides.

- Prevent asbestos dust being released or keep it to as low a level as possible.

- Make sure that it does not spread to affect other people.

- SOME GOOD WORK PRACTICES

You must not disturb any confirmed or presumed asbestos materials without an assessment being carried out by your manager or supervisor.

Some low risk materials can be worked on after an assessment and by following the precautions set out in the HSE’s ‘Asbestos Essentials – Task Manual’.  Some of the basic precautions required are:

- Keep asbestos materials (including waste) damp while you are working on them.

- Don’t use power tools on asbestos materials; they create dust.  Use hand tools instead.

- Use a suitable mask (respirator) when you can’t avoid creating dust.  Don’t breathe it in.  You can’t see harmful asbestos fibres when they are in the air.

- Clean up asbestos dust as you go with a ‘Type H’ vacuum cleaner. Don’t use brooms or brushes.

- Double bag asbestos waste before it dries in heavy duty polythene bags.  Label the outer one to show it contains asbestos.

- Wash before meals and before you go home.

- Don’t take protective clothing contaminated with asbestos home for washing: your boss should send it to a specialist laundry.

- FURTHER INFORMATION

Your Manager should provide you with proper information and training for working with asbestos.

Inserted Document

| Example Asbestos Management Plan (Asbestos present) |  |
| --- | --- |
| Example Asbestos Management Plan (No Asbestos) |  |
| Example LCC AMP letter (Post 2000 construction) |  |